SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across  networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I ___keerthana R (192524489)_________________ (Name / Reg No) certify that this submission is my original work and  that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the student: keerthanaR

Annexure A

Short Answer Test

1. A company needs to transfer a 150 MB design file from a client computer to a  remote server over a network with a bandwidth of 50 Mbps. The Transport Layer  divides the file into 1500-byte segments, while the Data Link Layer encapsulates  each segment into frames by adding 40 bytes of header and trailer. Calculate  the total number of segments and frames transmitted, the total transmission  overhead introduced by the Data Link Layer, and estimate the total transmission  time (ignoring propagation delay). Explain how the Transport and Data Link  layers ensure reliable communication.

ANS:Total Segments: 100,000 segments

  - Calculation: 150,000,000 bytes / 1,500 bytes/segment = 100,000 segments.

- Total Frames: 100,000 frames

  - Calculation: Since each segment is encapsulated into one frame, the number of frames equals the number of segments.

- Total Transmission Overhead: 4,000,000 bytes (4 MB)

  - Calculation: 100,000 frames × 40 bytes/frame = 4,000,000 bytes.

- Total Transmission Time: 24.64 seconds

  - Calculation:

    - Total data transmitted = 150 MB + 4 MB (overhead) = 154 MB.

    - Total bits = 154,000,000 bytes × 8 bits/byte = 1,232,000,000 bits.

    - Time = 1,232,000,000 bits / 50,000,000 bits/second = 24.64 seconds.

- Reliable Communication (Transport Layer):

  - Uses Sequence Numbers and Acknowledgements (ACKs) to ensure all segments arrive in order and verify receipt.

  - Implements Error Detection (e.g., checksums) to identify corrupted segments.

  - Uses Retransmission Mechanisms (such as Automatic Repeat Request - ARQ) to recover lost or damaged data.

- Reliable Communication (Data Link Layer):

  - Uses Frame Check Sequence (FCS) via Cyclic Redundancy Check (CRC) to detect bit errors caused by physical transmission.

  - Implements Flow Control mechanisms to prevent the sender from overwhelming the receiver's buffer.

  - Provides Error Correction by requesting retransmission of discarded, corrupted frames.

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each  frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate  the number of transmission windows required. If one frame in every window is  lost and must be retransmitted, determine the total number of retransmissions.  Explain how flow control improves communication efficiency.

ANS:Number of Transmission Windows Required: 8 windows

  - Calculation: 48 total frames / 6 frames per window = 8 windows.1

- Total Number of Retransmissions: 8 retransmissions

  - Calculation: Since 1 frame is lost in each of the 8 windows, 8 total frames must be retransmitted.1

- How Flow Control Improves Communication Efficiency:

  - Prevents Buffer Overflow: It ensures the sender does not transmit data faster than the receiver's buffer can process it, avoiding data loss due to congestion.

  - Optimizes Bandwidth: It regulates the rate of data flow to match network and receiver capacity, ensuring stable throughput.

  - Reduces Retransmissions: By preventing dropped packets caused by overflow, it eliminates the need for bandwidth-wasting retransmissions, leading to overall faster communication.1

3. A 12,000-byte IP packet is transmitted through a router where the Maximum  Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20- byte IP header, calculate the total number of fragments created and the total  header overhead introduced. Explain how the Network Layer ensures successful  delivery of fragmented packets to the destination.

ANS: total Number of Fragments: 9 fragments

  - Calculation: Each fragment has a payload capacity of 1,480 bytes (1,500 MTU - 20-byte IP header). 12,000 total bytes / 1,480 bytes per payload ≈ 8.1. Since you cannot send a partial fragment, 9 fragments are required.

- Total Header Overhead: 180 bytes

  - Calculation: 9 fragments × 20 bytes per IP header = 180 bytes total header overhead for the fragments.

- Network Layer Mechanism for Successful Delivery:

  - Identification Field: All fragments of the original packet share the same Identification number, allowing the destination to group them correctly.

  - Fragment Offset: Each fragment contains an offset value that indicates its specific position relative to the start of the original packet, which allows the destination to reassemble the data in the correct sequence.

  - Flags (More Fragments Bit): The network layer uses a "More Fragments" (MF) flag in the header. If the bit is 1, more fragments follow; if it is 0, it marks the last fragment, signaling to the receiver that the packet is complete and ready for reassembly.

4. A Transport Layer protocol uses a sliding window size of 6 frames. Each  frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate  the number of transmission windows required. If one frame in every window is  lost and must be retransmitted, determine the total number of retransmissions.  Explain how flow control improves communication efficiency.

ANS:

- Number of Transmission Windows Required: 8 windows

  - Calculation: 48 total frames / 6 frames per window = 8 windows.1

- Total Number of Retransmissions: 8 retransmissions

  - Calculation: Since 1 frame is lost in each of the 8 windows, 8 total frames must be retransmitted.1

- How Flow Control Improves Communication Efficiency:

  - Prevents Buffer Overflow: It ensures the sender does not transmit data faster than the receiver's buffer can process it, avoiding data loss due to congestion.1

  - Optimizes Bandwidth: It regulates the rate of data flow to match network and receiver capacity, ensuring stable throughput.1

  - Reduces Retransmissions: By preventing dropped packets caused by overflow, it eliminates the need for bandwidth-wasting retransmissions, leading to overall faster communication.1

5. A multimedia application transfers a 600 MB video file. The Session Layer  inserts a synchronization checkpoint after every 60 MB of transmitted data.  During transmission, a failure occurs after 390 MB has been transferred.  Determine the number of checkpoints created before failure and calculate the  amount of data that must be retransmitted after recovery. Explain the importance  of synchronization in the Session Layer.

- Number of Checkpoints Created: 6 checkpoints

  - Calculation: Checkpoints occur every 60 MB (60, 120, 180, 240, 300, 360). Since the failure occurred at 390 MB, the system successfully completed 6 checkpoints.

- Amount of Data to be Retransmitted: 30 MB

  - Calculation: Upon recovery, the system resumes from the last successful checkpoint at 360 MB. The data transmitted between the last checkpoint (360 MB) and the failure point (390 MB) must be retransmitted. (390 MB - 360 MB = 30 MB).

- Importance of Synchronization in the Session Layer:

  - Enables Error Recovery: Allows the application to resume data transmission from the last established checkpoint rather than restarting the entire 600 MB file transfer from the beginning.

  - Improves Efficiency: Minimizes wasted bandwidth and transmission time by preventing the need to re-send data that was already successfully transferred and acknowledged.

  - Ensures Continuity: Provides a mechanism to manage sessions effectively, ensuring that data integrity is maintained even when connection interruptions occur.

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%.  The compressed data is encrypted, adding 5% overhead to the compressed size.  Calculate the compressed file size, the encrypted file size, and the total percentage  reduction compared to the original file. Explain how the Presentation Layer  improves transmission efficiency and security.

ANS:Compressed File Size: 540 MB

  - Calculation: 900 MB - (40% of 900 MB) = 900 MB - 360 MB = 540 MB.

- Encrypted File Size: 567 MB

  - Calculation: 540 MB + (5% of 540 MB) = 540 MB + 27 MB = 567 MB.

- Total Percentage Reduction: 37%

  - Calculation: (Original Size - Encrypted Size) / Original Size = (900 MB - 567 MB) / 900 MB = 333 MB / 900 MB = 37%.

- Presentation Layer Improvements:

  - Transmission Efficiency: By compressing data before transmission, the layer reduces the actual amount of data being sent, which optimizes bandwidth utilization and significantly decreases transmission time.

  - Security: By encrypting the data, the layer ensures that even if intercepted, the information remains unreadable to unauthorized parties, thereby guaranteeing data confidentiality during transit.

7.An FTP application transfers 3 GB of data over a network with an effective  throughput of 120 Mbps. The Application Layer divides the data into requests of  3 MB each. Calculate the total number of requests generated and estimate the total  transmission time in seconds. Discuss how the Application Layer provides  reliable services to users.

ANS:

- Total Requests: 1,000 requests

  - Calculation: 3 GB = 3,000 MB. 3,000 MB / 3 MB per request = 1,000 requests.1

- Total Transmission Time: 200 seconds

  - Calculation: 3 GB = 3,000,000,000 bytes. 3,000,000,000 bytes × 8 bits/byte = 24,000,000,000 bits. 24,000,000,000 bits / 120,000,000 bits/sec (120 Mbps) = 200 seconds.1

- Application Layer Reliability:

  - Session Management: The layer establishes, manages, and terminates connections, ensuring that the user-to-application session remains stable throughout the data transfer.

  - Error Reporting: It interprets error messages from lower layers (like TCP) and provides meaningful status updates or error codes to the user, allowing for appropriate manual or automated intervention.

  - Data Verification: It often implements application-level checks, such as file integrity verification (using checksums or hashes), to ensure the received file matches the original after reconstruction.

  - Resume Capability: Many application-layer protocols (like FTP) support partial transfer recovery, allowing the application to restart a failed transfer from the last successful checkpoint instead of re-sending the entire file.

8.A packet experiences the following processing delays at different OSI layers:  Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data  Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms,  and the transmission delay is 12 ms. Calculate the total end-to-end delay  experienced by the packet and explain how each layer contributes to the  communication process.

ANS:Calculation of Total End-to-End Delay

- Total Processing Delay (Dproc​): Sum of the processing delays at all five specified OSI layers:
Dproc​=4 ms (Application)+3 ms (Transport)+5 ms (Network)+2 ms (Data Link)+1 ms (Physical)=15 ms

- Propagation Delay (Dprop​): Given as 18 ms

- Transmission Delay (Dtrans​): Given as 12 ms

- Total End-to-End Delay:
Total Delay=Dproc​+Dprop​+Dtrans​

- Total Delay=15 ms+18 ms+12 ms=45 ms

### 2. OSI Layer Contributions to the Communication Process

- Application Layer (4 ms): Provides the primary interface for network services (e.g., HTTP, FTP, or SMTP), allowing user applications to generate and format the raw data to be sent.

- Transport Layer (3 ms): Manages end-to-end communication, breaking large payloads into smaller segments (segmentation), assigning sequence numbers, and ensuring reliable, ordered data delivery through flow control and error recovery (like TCP).

- Network Layer (5 ms): Handles logical addressing (IP addresses) and path determination (routing), packaging transport segments into packets to guide them across multiple network nodes to their final destination.

- Data Link Layer (2 ms): Organizes network packets into frames, controls access to the physical transmission medium (MAC addressing), and adds error detection mechanisms (like CRC) to ensure reliable hop-to-hop delivery.

- Physical Layer (1 ms): Converts data frames into raw electrical, optical, or electromagnetic signals (bits) and transmits them across the physical medium (cables or wireless channels).

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where  each frame contains 80 bytes of protocol overhead added by different OSI layers.

Calculate the total number of frames required, the total overhead transmitted, and  the transmission efficiency as a percentage. Explain how protocol overhead  affects network performance.

ANS: Given:

- Useful data = 80 MB

- Frame size = 1600 bytes

- Protocol overhead per frame = 80 bytes

Step 1: Calculate Useful Data in Bytes

80 MB=80×1024×1024=83,886,080 bytes

Step 2: Calculate Payload (Useful Data per Frame)

Payload=1600-80=1520 bytes

Step 3: Calculate Number of Frames

Number of Frames=83,886,0801520=55,188.21

Since a partial frame cannot be sent,

Total Frames=55,189

Step 4: Calculate Total Overhead

Total Overhead=55,189×80=4,415,120 bytes
Total Overhead=4,415,120 bytes

Step 5: Calculate Transmission Efficiency

Efficiency=Useful DataUseful Data+Overhead×100
=83,886,08083,886,080+4,415,120×100
=83,886,08088,301,200×100=94.999%
Transmission Efficiency≈95%

Final Answer

- Total Number of Frames = 55,189

- Total Overhead Transmitted = 4,415,120 bytes

- Transmission Efficiency = 95%

Effect of Protocol Overhead on Network Performance

Protocol overhead consists of additional header and trailer bytes added by different OSI layers for addressing, error detection, and control. Although overhead is necessary for reliable communication, it reduces the amount of useful data in each frame. As protocol overhead increases, more bandwidth is consumed by control information, transmission efficiency decreases, and the overall network throughput is reduced

10.A hospital transfers a patient's medical record of 240 MB through an IoT enabled healthcare network. The Presentation Layer compresses the data by 30%,  the Transport Layer divides the compressed data into 1400-byte segments, and  the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the  compressed file size, the total number of segments transmitted, the total protocol  overhead introduced, and the final amount of data transmitted across the Physical  Layer. Explain how multiple OSI layers work together to ensure secure and  reliable communication.

ANS: Given:

- Original medical record size = 240 MB

- Compression by Presentation Layer = 30%

- Transport Layer segment size = 1400 bytes

- Data Link Layer overhead per frame = 60 bytes

Step 1: Calculate Compressed File Size

Original size:

240×1024×1024=251,658,240 bytes

Compressed size:

251,658,240×1-0.30=251,658,240×0.70=176,160,768 bytes
Compressed File Size=176,160,768 bytes  168 MB

Step 2: Calculate Total Number of Segments

Number of Segments=176,160,7681400=125,829.12

Since the last segment is partial,

Total Segments=125,830

Step 3: Calculate Total Protocol Overhead

Total Overhead=125,830×60=7,549,800 bytes
Total Protocol Overhead=7,549,800 bytes

Step 4: Calculate Final Data Transmitted Across the Physical Layer

Final Data=176,160,768+7,549,800=183,710,568 bytes
Final Data Transmitted=183,710,568 bytes

(≈ 175.20 MB)

Final Answer

- Compressed File Size = 168 MB (176,160,768 bytes)

- Total Number of Segments = 125,830

- Total Protocol Overhead = 7,549,800 bytes

- Final Data Transmitted Across the Physical Layer = 183,710,568 bytes (≈ 175.20 MB)

How Multiple OSI Layers Work Together

- Presentation Layer: Compresses the data by 30%, reducing the file size and improving transmission efficiency. It also performs encryption and data formatting when required.

- Transport Layer: Divides the compressed data into 1400-byte segments, provides end-to-end communication, error recovery, and reliable delivery.

- Data Link Layer: Adds 60 bytes of overhead to each frame for framing, error detection, and MAC addressing.

- Physical Layer: Transmits the final data (payload + overhead) over the communication medium.

Together, these OSI layers ensure that data is compressed, secure, reliable, error-free, and efficiently transmitted across the network.

RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25% | Demonstrates a thorough, accurate understandin g of concepts | Good understanding with minor gaps | Basic understanding with some misconception s | Poor understanding; major misconception s |
| Explanation | 25% | Detailed, well explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well structured answers with logical flow | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |

and

coherence

| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| --- | --- | --- | --- | --- | --- |
| Clarity | 5% | Clear, neat, professional presentation | Understandabl e with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |